Abstract043_Briano.doc

ConferenceTrack:  Homeland Security

Language:         English

Keywords:         Landmines detection, TNT fate nad transport

ContactTitle::    Dr.

ContactFirstName: Julio

ContactLastName:  Briano

University:       University of Puerto Rico

Web:

Position:         Professor

Country:          Puerto Rico

Email:            jbriano@uprm.edu

Fax:              787-834-3655

Submitted:        Friday, January 28, 2005, 11:20:00 AM

Title:

Transport of the Chemical Signature from Buried UXO and Landmines

Abstract:

The transport of the chemical signature compounds from buried UXO and landmines in a three-dimensional array has been numerically modeled using the finite-volume technique. A three-dimensional (3D) framework is required since two-dimensional (2D) symmetry may easily fade due to terrain topography: non-flat surfaces, soil heterogeneity, or underground fractures. Compounds such as trinitrotoluene, dinitrotoluene, and their degradation products, are semi-volatile and somewhat soluble in water. Furthermore, they can strongly adsorb to the soil and undergo chemical and biological degradation. Consequently, the spatial and temporal concentration distributions of such chemicals depend on the mobility of the water and gaseous phases, their molecular and mechanical diffusion, adsorption characteristics, soil water content and compaction, and environmental factors. We have studied the effect of soil type, terrain topography, in a homogeneous system, and have obtained the spatial and temporal distribution of the chemical-signature-compounds by solving the mass and energy transport equations, using the finite-volume technique. An interesting comparison between the transport in a terrain with a horizontal flat surface, and one with an incline surface, is presented.  The fact that the chemicals may migrate horizontally, giving higher surface concentrations at positions not directly on top of the objects, tells the need for understanding the transport mechanism if a chemical detector is to be used.  Surface concentrations decrease, when precipitation occurs due to advective flux around the object. Deformation in the concentrations contours after rainfall is observed in the inclined surface and it is attributed to both: the advective flux, and to the water flux at the surface caused by the inclination.

MailingAddress:

Julio G. Briano

Chemical engineering department

University of Puerto Rico

Mayagüez Campus

Mayagüez, puerto Rico 00681-9046

Phone: +1 (787) 832-4040 ext2545, 2592, 2593

Authors:

Julio G. Briano, jbriano@uprm.edu, University of Puerto Rico at Mayagüez,(P)

Ernesto E. Borreroa, ernestoborrero@hotmail.com, Cornell University

Maik Irrazábal, mairrazabal@hotmail.com, University of Puerto Rico at Mayagüez

Miguel Castro, mcastro@uprm.edu, University of Puerto Rico at Mayagüez

Samuel P. Hernández, sp_hernandez_uprm@yahoo.com, University of Puerto Rico at Mayagüez