Fifth LACCEI International Latin American and Caribbean Conference for Engineering and Technology (LACCEI’2007)

“Developing Entrepreneurial Engineers for the Sustainable Growth of Latin America and the Caribbean:

Education, Innovation, Technology and Practice”

29 May  – 1 June  2007, Tampico, México.

##### Stochastic Analysis of Operations Decoupling in a Flexible Manufacturing System

G. Pérez-Lechuga, glechuga2004@hotmail.com,

Miriam Álvarez Suárez, miriam@uaeh.edu.mx,

Manuel González Hernández, mghdz@uaeh.edu.mx,

J. Manuel Saucedo-Solorio, sausedo@uaeh.edu.mx

Centro de Investigación Avanzada en Ingeniería Industrial

Universidad Autónoma del Estado de Hidalgo

ICBI, CIAII, Carretera Pachuca-Tulancingo Km.4.5, Pachuca Hgo, México

Resumen

En los sistemas de produccion en serie, un almacenamiento se puede puede proveer entre los procesos para evitar la interferencia devida a la perdida de sincronizacion. Para manufacturar un producto, el proceso se divide en tareas indioviduales, en subprocesos de ensamble, tales tareas son independientes y deben ser coordinadas. Para reducir la interdependencia  entre operaciones y para mantener la salida de la linea de produccion es comun introducir almacenamientos intermedios entre las operaciones. Estos almacenamientos desacoplan las operaciones y eliminan tal interdependencia excepto en los casos cuando un almacenamiento se vacia en los procesos previos. En este trabajo se estudia flujos con almacenamiento de material en un sistema flexible de manufactura considereando dos maquinas. Se desarrollan procedimientos para calcular medidas de desempeño de estado estable, incluyendo las perdidas por interferencia y algunas distrucines limite usando procesos de Markov.

Palabras clave: Sistemas flexibles de manufactura, Procesos de Harkov, Sistemas dinámicos.

Abstract

In serial production systems, storage may be provided between processes to avoid interference due to lack of synchronization. In order to manufacture a product, a job is divided into individual tasks, typically manufacturing or assembly processes. These tasks are interdependent and should be coordinated. To reduce the interdependence between downstream and upstream operations and to maintain the output of the production line, it is common to introduce buffers between the operations. These buffers decouple operations and eliminate the interdependency unless the buffer is emptied when a shutdown occurs upstream. In this paper we study the buffered flows of matter in a flexible manufacturing system considering only two machines. We develop procedures to compute some steady state performance measures, including the interference loss and some limiting distributions. We use the Markov processes theory to obtain our results.

Keywords: Flexible Manufacturing Systems, Markov Processes, Performance Evaluation, Dynamic Systems.

- Introduction

The dynamics of continuous systems are often modeled by a set of differential equations that can express the relationships between rates of changes in the values of system state variables. Given an initial state and boundary conditions, these equations completely specify a model of the system’s dynamic behavior. When this system of differential equations is particularly simple or has some special properties, it can be solved analytically to find the system path of motion (trajectory). However, many interesting models are too complex to solve analytically and must be simulated by numerically integrating the set of differential equations (Schruben, et al., 2000). If the system is modeled using random processes, then the simulations can be used to generate sample paths for statistical analysis.

Flexible manufacturing systems (FMS) are an important class of discrete event dynamic systems. Flexibility means to produce reasonably priced customized products of high quality that can be quickly delivered to customers. Flexible manufacturing systems are computer hanced batch or repetitive processes that facilitate the production of high volumes of customized products on highly automated equipment that is responsive to software instructions. An FMS is a queueing network system where different classes of products are processed contemporaneously. Each product has to perform its own orderly sequence of operations, different for each class, in order to be completed. The same machine can perform operations on different product classes, eventually with different service times: the same operation can be performed on alternative machines. In this sense, flexibility is the capability of the FMS to cope in the time with changing product class blend and production inconveniences such as buffer blockages and machine breakdowns, maintaining optimum production target, machine load balance and, if required, an assigned production mix (Balduzzi, et al., 1998). Although numerous benefits are associated with automated flexible manufacturing processes, such as reduced labor cost, faster throughput times and faster responses to demand volume changes and to product design changes, the decision whether to optimize such a process is difficult.

In practice, there is major uncertainty about implementation costs, date of on-line availability, and performance characteristics once on line. Many of the benefits typically associated with flexibility, such as improved quality control, reduced work-in-process inventories, and reduced lead times, are not yet fully substantiated and may be difficult to measure. In this paper, we develop a set of performance measures to evaluate the dynamic behavior of an FMS considering the simplest form of the system: two machines and a buffer for operations decoupling.

- Background

The scheduling problems encountered in an FMS can be separated into several distinct types which encompass a wide range of resources including parts, robots, machines, and AGVs. Stecke (Stecke, et al., 1981), categorize different scheduling problems and apply sets of dispatching rules to each problem in an effort to evaluate the impact of various rules on the system performance. Several researchers have since evaluated different problems under different sets of rules. Dar-El (Dar-El, et al., 1982) evaluated the impact of a “good” schedule for a particular problem and the effect of any one dispatching rule has been found to vary with several factors such as system layout, system state, and the desired performance measure. Other researchers (Cho, et al., 1993, Jones, et al., 1995); have suggested using neural networks to identify candidate rules for multi-pass simulation analysis. This author takes into account multi-criteria performance measures. When a new schedule is desired, a neural net generates good rules for each performance measure and then simulation is used to predict how each rule does against all performance measures simultaneously. In both cases, the neural network is trained off-line by the simulation under a variety of input conditions. They also examine the aplications of discrete-event simulation for floor control of a flexible manufacturing systems.

Cho (Cho, 1992) defines five types of scheduling problems in the context of an automated workstation. At each decision, the neural network generates candidate rules for each problem type and these rules are then evaluated throught simulation. The analysis of a single lines that ivolves Markov models has been suggested by Hongler (Hongler, 1996), and Barucha-Reid (Barucha-Reid, 1997).

- Mathematical model

A stochastic process X(t);  t≥0 is regenerative if there exist a sequence β0, β1 of stopping (regenerative) times such that:

1.  T=Tl;l=0,1,... is a renewal process

2. For any l, m∈0,1,..., t1,...,tl>0, and any bounded measurable function  f:Il→R

EfXβm+t1,...,Xβm+tl|Xk,K≤βm=

ΕfXβt1,...,Xβtl,                            (1)

A stochastic processes M(n)n=0,1... with finite state space  Z=z1,z2,...,zs is called a Markov chain (with discrete time), if for all  n∈N and all w0,...,wn∈Z

PM0=w0,M1=w1,...,Mn=wn

=PM0=w0PMi=wi|Mi−1=wi−1,                    (2)

The process is called a homogeneus Markov chain if there is a s×s-matrix Q=pij such that

PM(n)=zj|Mn−1=zi=pij,∀n.                    (3)

Our general model is defined following the ideas developed by Hongler (Hongler, 1996).We supose that the dynamics of the system is given by the simple production line schematically represented in figure 1.

## Figure 1: Two machine production line.

Two failure prone machines M1 and M2 are partly decoupled by the introduction of a buffer B12 which has a maximum capacity equals to ϕ[parts]. The mean time to failure and the mean time to repair will be denoted respectively by λj−1 and  μj−1 for the machineMj, j=1,2. In this model, ρj=λjμj−1 represents the indisposability factor of   Mj. The production rate of Mj is  [parts/unit time]. The time dependent content of the bufferB12, Mt, t≥0 can be considered as a random variable in the interval −ϕ2,ϕ2. We define the derivated stochastic process:

(4)

where for j=1,2,

(5)

The waiting time intervals between transitions from states {0} to {1} and vice versa are characterized respectively by probability distributions  ψjz and φjz on positive random variables. Thus we have that

zdφz=λj−1, and  zdψz=μj−1,

In this model we are interested in the case where ψz and φz  are exponentially functions distributed.

Theorem 1: Let πt,t≥0 be the stochastic process defined in (5). Since φz and ψz have finite means, and  φz + ψz has a continuous distribution, then

limt→∞Pπt=1=μμ+λ,

limt→∞Pπt=0=μμ+λ,

Proof: See (Pérez-Lechuga et al., 2004), see also (Parzen, 1999) for a more widespread proof.

Theorem 2: Let the stochastic process defined in (5), then for any s, t≥0, the transition probability functions,  pjkt=Pπt+s=k|πs=j  are given by

p00(t)λμ+λ+μμ+λe−μ+λtp10(t)λμ+λ1−e−μ+λtp01(t)μμ+λ1−e−μ+λtp11(t)μμ+λ+λμ+λe−μ+λt                   (8)

Proof: The transition probabilities of process (5) can be obtained using the forward Kolmogorov differential equations

∂∂tpjkt=−qkpjkt+i≠kpjitqik,

Where qjt  and qjkt are the homogeneous intensity of passage and the homogeneous intensity of transition respectively, and pjkt is the transition probability function.

pjkt=Pπt+s=k|πs=j.

Let the intensities of passage from 0 to 1 be given respectively by q0=μ and q1=λ. It then follows that the transition intensities are given by q01=μ and q10=λ.

The Kolmogorov differential equations (9) then become

∂∂tp00(t)−μp00(t)+λp01(t)∂∂tp01(t)−λp01(t)+μp00(t)∂∂tp11(t)−λp11(t)+μp10(t)∂∂tp10(t)−μp10(t)+λp11(t)                  (10)

Since p01t=1−p00t then, the first of these equations can be rewritten (Parzen[11])

∂∂tp00t=−μ+λp00t+λ,                    (11)

Equation (11) is an ordinary differential equation of the form (with gt=p00t, v=μ+λ, ht=λ)

g't=−vgt+ht,   a≤t<∞

whose general solution is

gt=e−vt+shsds+e−vt−aga

Then using the boundary condition p000=1 we obtain

p00t=λe−μ+λt−sds+e−μ+λt,             (12)

Using equation (12) it follows the proposed results.

Corollary 1: Let  p0=Pπ0=0 then

Eπt=μμ+λ−p0−λμ+λe−μ+λt

Corollary 2: For the Markov chain defined in (4) we have:

(13)

Let us define the loss of production due to the period the machines have to wait for service as the machine interference.

Let the random variable W(t) denote the number of machines not working at time t and let

limt→∞PWt=w,    w=0,1...,m

where m is the number of machines in the system (2 in this case). Using (6) and (7)

as estimators of the limiting distribution, and by the independence hypothesis between machines we have

=limt→∞Pπ1t=1∩π2t=1=μ2μ+λ2

and

=limt→∞Pπ1t=0∩π2t=1+π1t=1∩π2t=0=2μλμ+λ2

Let α, β, and γ denote the average number of machines working, being serviced, and waiting to be serviced, respectively.We have the following identities (Barucha-Reid, 1997):

α+β+γ=m                       (14)

αβ=μλ                                (15)

β=r−w=0r−1r−wπw            (16)

where r denotes to the number of repairmen assigned to the system.

Equation (16) relates to the equality of the number of engaged repairmen and the number of machines being serviced. From (14) note that

γ=m−μ+λλr−w=0r−1r−wπw            (17)

Thus, for m = r = 2

γ=m−μ+λλr−r+r−1=0

Similarly, for the case of one repairman and m = 2,

γ=2−μ+λλ1−π0=2−μ+λλ

Referencias

1. Schruben, L. W., Mathematical Programming Models of Discrete Event System Dynamics, Proceedings of the 2000 Winter Conference, J.A. Joines, R.R. Barton, K. Kang, and P.A. Fishwick, Eds. 2000.

2. Balduzzi, F. and Menga, G., “A state variable model for the fluid approximation of flexible manufacturing systems”, Proc. IEEE, International Conferences on Robotic & Automation, Leuven, Belgium., 1998.

3. Stecke, K. E., and J. J. Solberg. 1981. “Loading and Control Procedures for Flexible Manufacturing Systems”, International Journal of Production Research, No. 19, 481-490.

4. Dar-El, E. M., and R. A. Wysk. 1982. “Job Shop Scheduling - A Systematic Approach”. Journal of Manufacturing Systems, Vol. 1, 77-88.

5. Cho, H., and R. A. Wysk. 1993. “A Robust Adaptive Scheduler for an Intelligent Workstation Controller”. International Journal of Production Research, Vol. 31, No. 4, 771-789.

6. Jones, A., Rabelo, L., and Y. Yuehwern. 1995. “A Hybrid Approach for Real-Time Sequencing and Scheduling”. International Journal of Computer Integrated Manufacturing, Vol. 8. No 2, 145-154.

7. Cho, H., An Intelligent Work-station Controller for Computer Integrated Manufacturing, Ph.D. Disertation, Industrial Engineering, Texas A & M, College Station, TX . 1992.

8. Hongler, M.-O., Chaotic and Stochastic Behaviour in Automatic Production Lines, Springer Verlag, Heidelberg Germany, 1996.

9. Barucha-Reid, A.T., Elements of the Theory of Markov Processes and their Applications, Dover Publications, 1997. 429-438.

10. Pérez-Lechuga, G., Alvarez-Su´arez, M., Pla-Aragones, L.M., Muñoz-Orán, M.G., “A Mathematical Characterization of the Performance in a Conwip Line as a Function of its Maintenance”, WSEAS Transactions on Systems, ISSUE 3, Vol 3, 1176-1181, 2004.

11. Parzen, E. Stochastic Process, Classics in Applied Mathematics, SIAM. Philadelphia PA. 1999.

- Autorización y Renuncia

- Los autores authorizan a LACCEI para publicar el escrito en los procedimientos de la conferencia. LACCEI o los editors no son responsables ni por el contenido ni por las implicaciones de lo que esta expresado en el escrito

- Authorization and Disclaimer

Authors authorize LACCEI to publish the paper in the conference proceedings.  Neither LACCEI nor the editors are responsible either for the content or for the implications of what is expressed in the paper.